Head Trauma App Design Plan

Users: military medics/EMTs in a triage scenario

- Open app to homescreen: Table view or collection view for all your patients, with an “add new” button for new patient

  - Sidebar option to add a new patient when in a single patient view

- New patient initial screen: open to questions (based off the Glasgow coma scale)

  - Design ideas: table of questions that change color (one tap = green, two taps =  red)

  - Or Y/N on side that turns green or red

  - Or phrase question to favor one response and tap to color the box if it’s true

    - Will design algorithm later that takes answers into the R/Y/G status

    - Rank questions according to level of emergency with the most time sensitive first

- Take photo screen

  - To track patient

  - Also for potential machine learning app capabilities later

- Bluetooth: Try to automatically connect to bluetooth bandage as soon as app opens, (run a loading tab animation at the bottom)

  - If connected: turn the background red/yellow/green based on vitals

    - .visible = false for “CONNECT” button

  - If not connected: give option to retry bluetooth connection or cancel bluetooth

  - Pull up icons for pulse ox, heart rate and temperature as soon as bandage bluetooth connects

    - Have them white when not connected and then change color based on the vital info

- Once done with patient, option to return to gallery mode to view all patients that have been logged or check in another patient

Back end 1:

Blue light flashes while connecting

Audio: “Connected” or “Secure bandage and pull battery tab”; 30 s later. If still bleeding, apply direct pressure and hold until bleeding stops

Sensor begins monitoring… Parameters for sensor:

|  | O2 | HR | TEMP |
| --- | --- | --- | --- |
| RED | <87% | <45, >110 | <95.9;>101 |
| YELLOW | 88-90% | 46-55; 91-109 | 96-97.4; 99.5-100.9 |
| GREEN | >91% | 56-90 | 97.5-99.4 |

Acquires GPS

Back End 2:

Sensor detects vital signs and calibrates sensor light based on parameters

Lights flash in between Red/Yellow/Green while detecting parameters

Audio:

If Red: “Call 911 and continue to monitor”

“check for pulse”—Start CPR

If Yellow: Continue to monitor

Green: Make sure bleeding stopped and continue to monitor

____________

RED

Low Heart

& Oxygen

Done

Questions

Able to:

OPEN EYES: Y/N

SPEAK : Y/N

MOVE BODY: Y/N

More q’s as needed……

** Make updated images for storyboards 2-6

Back End 3:

Algorithm:

AI to assess:

*Age (baby, child, adult, geriatric

* Bleeding head

*eyes open/closed

*Male / or female

Weather calibration if outside

GPS tracking

Face Photo

Are you outdoors/indoors?

NEXT or SKIP

Tap to take photo

Accept/Retake

RED

Sends score to ER/EMS

StatusUpdate

HERO ID 001

RED

Adult male, bleeding from head with dropping HR, O2

Risk 5

Indoors

Location Link

Back End 4:

If RED: Questions:

Vomiting? YES/No

Head Bleeding? YES/NO

Skull fracture? YES/NO

“Call 911, apply direct pressure

Turn on Left side”

If Green, continue to monitor

YELLOW

Heart Rate improved

Able to:

Y/N:OPEN EYES

Y/N: SPEAK

Y/N: MOVE BODY

NEXT